# SKY130 Area Estimator: Model Comparison

# 1. The Old Model (Flawed)

The previous script fitted a simple linear regression to calculate the layout area of MOSFETs:

    Area = a*(W * nf) + b*L + c

This model is mathematically incomplete. In a physical Magic layout, the overall area is a rectangle defined by its bounding box (Width × Height). Height is proportional to W (due to source/drain extensions), while Width is proportional to (L * nf) (due to multiple gates). Therefore, the true Area equation MUST have a multiplication cross-term (W × L), which the simple linear model lacked.

# 2. The New Model (Accurate Bounding-Box)

To solve this, the new script separately models the bounding box dimensions based on the device_db data:

    Height = ah*W + bh
    Width = aw*(L * nf) + bw*nf + cw

Then, the final Area is calculated properly as:

    Area = Height × Width

This correctly captures the cross-multiplication term, allowing accurate scaling for devices that are simultaneously wide and long.

# 3. Numerical Error Comparison (nfet_01v8)

| Dimensions | Actual Area (µm²) | Old Model Prediction | New Model Prediction |
| --- | --- | --- | --- |
| W=0.42 L=0.15 nf=1 | 5.32 | 6.06 (14.0% err) | 5.34 (0.5% err) |
| W=1.00 L=0.15 nf=1 | 6.54 | 6.68 (2.1% err) | 6.57 (0.5% err) |
| W=2.00 L=0.15 nf=4 | 14.72 | 14.11 (4.1% err) | 14.72 (0.0% err) |
| W=2.00 L=0.35 nf=2 | 12.10 | 10.78 (10.8% err) | 12.19 (0.8% err) |
| W=0.42 L=0.50 nf=1 | 6.20 | 7.67 (23.8% err) | 6.15 (0.9% err) |
| W=1.00 L=0.50 nf=2 | 10.07 | 9.35 (7.2% err) | 10.07 (0.1% err) |

As shown above, the new model reduces worst-case estimation errors from nearly 24% down to under 1%.

# 4. Parity Plots

The plots below show Actual Area (X-axis) vs Predicted Area (Y-axis). A perfect model lies exactly on the dashed black line. The new model (blue circles) follows the ideal line perfectly, whereas the old model (red crosses) diverges heavily.